KiN

Draft Tech Rider 2026.1

NOTES ABOUT THIS DOCUMENT

The following document sets out the Preliminary Technical Requirements of the Production and the support to be provided to the Company by the Presenter.

Understandably, sites and circumstances of the performances will differ, so we welcome suggestions or solutions to our requirements from individual Presenters and Festivals.

COMPANY CONTACTS

Isha Sharvani (Producer)   			ishasharvani@gmail.com

Devissaro (Tech. Director) 			devissaro@gmail.com

Stephen Carr (Production/Stage Manager)	darksidetheatrics@outlook.com

1.  PERFORMANCE DESCRIPTION

KiN is a show that combines dance, aerial acts, live music, video projections as well as live music and recorded backing tracks. Ideally suited for a large proscenium stage with a fly tower and raked auditorium, the company however is able and willing to adapt the show for other types of performance venues.

KiN is a family friendly show, suitable for an audience between 300 and 1200 people. It is a compact show in terms of personnel and baggage, making it an ideal touring production.

This Technical Rider assumes that the venue has a safe, central rigging point or aerial truss for hanging the Mallakhamb rope for the aerial acts that are an essential part of the show. KiN however is adaptable, and has been performed using a sectioned aluminium truss mounted onstage. For venue specific rigging options or queries, please contact us to discuss further.

PERSONNEL

### Touring Personnel

### 4x adult Performers

### 1x child Performer

### 1x Technical Director

### 1x Production/Stage Manager

  - 1x Community Engagement Artist

Total    8 persons

Personnel provided by Venue

  - 1x Lighting Board/Projector programmer

  - 1x Sound Board operator

  - 1x Stage Hand/Rigger (props and fly rail)

Total   3 persons

2.  SET OVERVIEW

All tour set elements are provided by the touring company. They comprise:

- Two Projection Scrims

- 1x full stage-width scrim hung upstage, approx. 1.5m in front of the cyclorama.

- 1x smaller scrim downstage OP. Both scrims fly in and out during the show

- 1x Domestic Set downstage of the proscenium (OP) comprising

    - a turkish carpet

    - low table and stool

    - a coloured glass lamp shade that can be lowered in.

Approx. area 1.8m X 2.4m

3.  1x Musician Set downstage of proscenium (PS) comprising

    - a Turkish carpet

      - Assorted instruments

    - Sound requirements see below

Approx. area 1.8m X 2.4m

## 3.  STAGING

The below are ideal stage dimensions to ensure the quality of presentation as well as performer safety, however all stage dimensions can be negotiated.

- Performance area: 12m wide, 12m deep (less also possible)

- Additional width for lighting booms: 2m each side of performance area

- Proscenium width: 12m (less also possible)

- Proscenium height: 6-8m

- Height for lighting bars: 6-8m

- Flat, smooth stage floor with no rake or impairment

- Stage floor black, ideally sprung and covered in tarket

#### Presenter to supply:

- Venue plans and current technical specifications.

- Full stage-width Cyclorama across the rear stage (ideally)

- Aerial acts are integral to the show. For this we require a safe and rigid centre-stage attachment point/aerial truss at a height of 5.5 - 7.5 meters, for hanging a fixed aerial rope from which a dancer can be suspended.

- Qualified Rigger may be required to sign off on the rigging

- Appropriate ladders/ EWP access for overhead access.

#### Touring Company to supply:

- Aerial apparatus and harnesses

- Stage and Set plan

4. LIGHTING

Venue stock will be used where possible, however additional hires may be necessary.

#### Venue to provide: the following systems of lighting, to cover the entire stage:

1. FOH face fills and DS area spots

2. Multiple colour Back Wash preferably provided by RGBW LED washes/ Moving Head washes

3. Moving Head spots for full stage gobos and special fx

4. High Sides/Pipe Ends Stage Left and Right. Ideally provided by

programmable lights such as RGBW LED pars

5. SIDE LIGHTS on booms in each wing

• Shins: Stage Left and Right: Profiles on Booms (approx 30cm from floor)

• Mids: Stage Left and Right: Fresnels on Booms (approx 1.2m from floor)

• Heads: Stage Left and Right: Profiles with barn doors on Booms (approx 2m from floor)

6. 2 banks of LED cyc floods set up to independently light the cyclorama as well  as and the scrim in front of it.

7. Around 20 instruments (ideally profiles) to be used as specials.

8. Full black-out

#### Touring company to provide:

  - Venue specific lighting plan based on venue-provided stage and standard lighting plans.

  - Laptops to run light cues, backing audio tracks and AVs

  - Any show specific set electrics & relevant cabling

5. VIDEO PROJECTIONS

The projections, which include recorded AV as well as live feed, are projected on two scrims provided by us - one 9m wide, positioned upstage in front of the cyclorama, the other a smaller scrim positioned downstage OP. Both scrims fly in/out during the show.

Venue to provide:

  - 10-20K lumens short throw projector mounted above stage for projecting video across the full rear cyclorama/scrim. Ideally it should be positioned upstage of the aerial bar to eliminate shadows during aerial acts. This projector will need to be able to switch between two inputs; AV from a laptop at the control desk, and live feed from a camcorder mounted offstage.

  - 6-10K lumens short throw projector placed on the downstage floor OP for projections on the smaller scrim. This projector can be hired should the venue not have one.

#### Touring company to provide:

- Camcorder, tripod and 50m HDMI cable to connect live feed camera to the main projector mounted above the stage.

- Laptops for AV, positioned at the SM location or the FOH desk.

6. SOUND REQUIREMENTS

The music for the show is a combination of recorded music, AV projections, onstage speech and live music provided by one musician. The musician has a fixed area for music (extreme downstage PS/on the proscenium apron)), but also performs across the stage.

Music includes live vocals with piano/keyboard, electronics and percussion - on an eclectic range of instruments including assorted drums, hand percussion, Tibetans bowls and santoor (Indian zither).

The  dancers are also part of the aural soundscape; their voices, footwork, body rhythms and vocal percussion also need to be mic’d.

Recorded music, backing tracks and AV are played from Qlab.

Venue to provide:

      - 4x onstage monitors for dancers.

  - 1x Separate monitor (concealed, DS PS) for the musician. This is especially important as live music has to be synchronised with backing recorded tracks.

  - 88 note weighted keyboard

  - Audio cabling and stereo ep (mini jack) to xlr/phono adaptors depending on mixer input.

  - Multi-channel mixer, preferably digital, with min 3-4 aux channels and FX.

  - Wireless Headset communication between FOH and backstage (light/sound/projector  operators and Stage Manager).

Microphones

- 8-10  mics for the musicians area downstage PS

- 4 Border mikes for downstage footwork

- 2-4 Rifle or boom mics for picking up performers onstage voice etc.

- 2 cordless mics for announcements etc.

#### Touring company to provide:

  - All instruments except keyboard

  - AV, backing tracks and recorded music

7. FX

Venue to provide:

▪ 2 Smoke machines (not just haze) with DMX control, operated from the control board.

### Control desks for Light and Sound must be placed together within the auditorium with a clear view of the stage.

8. TIMELINE

Bump In Time

We require the venue with technicians/stage crew from at least one day (ideally more) prior to the first show day.

Day 1

4hr call 	- install truss/rigging points (ideally pre-rigged)

- install light specials

- install scrims and stage sets

- install instruments and mics

- rig and test projectors

LUNCH

4hr call 	- Plot lighting cues

- mic and sound check with musician

DINNER

4hr call 	- Lay tarket

- Tech. rehearsal with performers

Day 2

4hr call	- dancers warm-up onstage

- full tech rehearsal

LUNCH

4 hr call	- corrections and adjustments from morning rehearsals

- dancers pre show warm-up onstage

EVENING	- PERFORMANCE

### Bump Out Time

- 3 hours after show